Reliance Foundry Co Ltd

#207-6450 148 St

Surrey, BC V3S 7G7

Phone: 604-547-0460

Phone: 1-877-789-3245

Fax: 604 590-8875

Email: info@reliance-foundry.com

Website: www.reliance-foundry.com

Reliance Foundry is an award-winning supplier of high-quality stock and custom-designed site furnishings

for architectural, traffic management, and industrial applications. Reliance Foundry brings over 90 years of expertise to the design and manufacturing of our products.

Reliance Foundry detectable warning plates are ground surface indicators that assist the visually impaired to safely navigate the surrounding environment. Detectable warning plates are made from durable cast iron, proven to withstand harsh climates, vehicular traffic including snow plows, and everyday urban wear and tear. Easily installs into prepared concrete for sidewalks, stairs, and platforms.

Specification Coordination: Edit this guide specification to meet project requirements. Coordinate with other specification sections as required. If using more than one detectable warning plate type, use the same designation in both drawings and specifications. Display hidden text to see or print notes to specifier.

Drawing Coordination: Show locations, sizes, layout, and spacing of each type of detectable warning plate. Soft metric conversion is provided for all measurements.

SECTION 32 17 00

DETECTABLE WARNING PLATES

GENERAL

SUMMARY

Section Includes: Detectable warning plates for paving applications.

Related Requirements:

Section 32 13 13.17 – Roller-Compacted Concrete Paving.

Section 32 14 23 – Asphalt Unit Paving.

REFERENCE STANDARDS

ADA-ABA Accessibility Guidelines for Buildings and Facilities (ADAAG)

ADAAG 4.29.2 - Detectable Warnings on Walking Surfaces.

Americans with Disabilities Act (ADA)

ADA 705.1.1 - Detectable Warnings; Dome Size.

ADA 705.1.2 - Detectable Warnings; Dome Spacing.

ADA 705.1.3 - Detectable Warnings; Contrast.

ADA 705.2 - Detectable Warnings; Platform Edges.

ASTM International (ASTM)

ASTM A48 - Standard Specification for Gray Iron Castings.

International Code Council (ICC)

ICC A117.1 - Accessible and Usable Buildings and Facilities.

American Concrete Institute (ACC)

ACI 530.1 - Specification for Masonry Structures.

American Society of Civil Engineers (ASCE)

ASCE 6 - Building Code Requirements and Specification for Masonry Structures and Related Commentaries.

The Masonry Society (TMS)

TMS 402/602 - Building Code Requirements and Specification for Masonry Structures.

SUBMITTALS

Comply with Section [01 33 00 – Submittal Procedures.] [_________.]

Delete the paragraph below if specifying colors in this section or in Drawings.

Reliance's bicycle storage units may contribute to these and other LEED credits. Adjust the list below to reflect credits sought for the project.

Credits below are based on LEED 2009 New Construction, Core and Shell, and Schools rating systems.

Product Data: Provide for each type of product used, including:

Preparation instructions and recommendations

Storage and handling requirements and recommendations

Installation methods

Color Samples: [Submit manufacturer’s standard colors for selection.] [Submit sample of color specified.]

Manufacturer's Maintenance Instructions for cleaning and maintenance.

Warranty Documentation: Submit sample of manufacturer’s warranty.

QUALITY ASSURANCE

Manufacturer Qualifications: Minimum 5-year experience manufacturing similar products

Installer Qualifications: Minimum 2-year experience installing similar products.

Mockups: Verify selections made under sample submittals, demonstrate aesthetic effects, and set quality standards for materials and execution.

Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

Source Limitations: Single source detectable warning plates and integrated anchoring materials.

DELIVERY, STORAGE, AND HANDLING

Deliver and store products in manufacturer’s unopened packaging bearing the brand name and manufacturer’s identification until ready for installation.

Handle materials to avoid damage.

SEQUENCING

Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

WARRANTY

See http://www.reliance-foundry.com/site/documents/warranty-bollards-and-bike-parking-products.pdf for warranty information.

        - Manufacturer's Warranty: Five-year warranty from date of Substantial Completion.

PRODUCTS

MANUFACTURERS

Manufacturer: Reliance Foundry Co. Ltd.

Phone: 604-547-0460 or 1-877-789-3245

Fax: 604-590-8875

Website: https://www.reliance-foundry.com/

Email: info@reliance-foundry.com

Substitutions: [Comply with provisions of [Section 01 25 00 "Substitution Procedures"] [_________] for substitution procedures.] [Not allowed.]

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 – Product Requirements.

DETECTABLE WARNING PLATES

Material:

Cast iron conforming to ASTM A48, Class 30B, minimum.

Installation: Direct Embed.

Quick Connect Plates [Designer’s Designation]

Model: Reliance Foundry

Finish: Manufactured true to pattern; component parts fit together in satisfactory manner. Uniform quality free of blowholes, porosity, hard spots, shrinkage distortion or other defects. Cleaned by blasting.

Visual warning surface to remain raw/unpainted.

Polyester powder coat over epoxy primer: [Safety yellow].

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4984-24Q.

Length: 24 inches (60.7 cm)

Width: 24 inches (60.7 cm)

Weight: 52 pounds (23.6 kg)

Design: Square

Material: Cast iron

Country of Origin: USA

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4984-30Q.

Length: 30 inches (76.2 cm)

Width: 24 inches (60.7 cm)

Weight: 75 pounds (34 kg)

Design: Rectangular

Material: Cast iron

Country of Origin: USA

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4984-36Q.

Length: 36 inches (91.4 cm)

Width: 24 inches (60.7 cm)

Weight: 109 pounds (50kg)

Design: Rectangular

Material: Cast iron

Country of Origin: USA

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-24

Height: 1.47 inches (3.73 cm)

Length: 24 inches (60.7 cm)

Width: 24 inches (60.7 cm)

Weight: 52 pounds (23.6 kg)

Design: Square

Material: Gray cast iron

Country of Origin: India.

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-15R

Height: 1.54 inches (3.91 cm)

Length: 24.2 inches (61.47 cm)

Width: 18.9 inches (48 cm)

Weight: 47 pounds (21.32 kg)

Design: Radial Plate

Material: Gray cast iron

Country of Origin: India.

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-20R

Height: 1.54 inches (3.91 cm)

Length: 24.2 inches (61.47 cm)

Width: 18.9 inches (48 cm)

Weight: 45 pounds (20.41 kg)

Design: Radial Plate

Material: Gray cast iron

Country of Origin: India.

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-18

Height: 1.47 inches (3.73 cm)

Length: 24 inches (60.7 cm)

Width: 18 inches (45.72 cm)

Weight: 45 pounds (20.41 kg)

Design: Rectangular

Material: Gray cast iron

Country of Origin: India

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-30

Height: 1.47 inches (3.73 cm)

Length: 24 inches (60.7 cm)

Width: 30.31 inches (76.99 cm)

Weight: 73 pounds (33.11 kg)

Design: Rectangular

Material: Gray cast iron

Country of Origin: India

Installation: Set in place

Detectable Warning Plate [Designer’s Designation]:

Model: Reliance Foundry; R-4985-25R

Height: 1.47 inches (3.73 cm)

Length: 24.2 inches (61.47 cm)

Width: 20.6 inches (52.32 cm)

Weight: 48 pounds (21.77 kg)

Design: Radial Plate

Material: Gray cast iron

Country of Origin: India

Installation: Set in place

STANDARDS COMPLIANCE

U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines for Buildings and Facilities.

ICC A117.1 - Accessible and Usable Buildings and Facilities.

VISUAL CONTRAST

Standards Compliance: ADAAG 2.29.2 and ADA 705.1.3.

Visual surfaces to contrast visually with adjacent walking surfaces; either light-on-dark or dark-on-light.

COEFFICIENT OF FRICTION

Comply with all ADA and ABA guidelines. The measured coefficient of friction is 0.9.

DOME SIZE

Standards Compliance: ADAAG 2.29.2 and ADA 705.1.1.

Base diameter: From 0.9 to 1.4 inch (2.3 to 3.6 cm).

Top Diameter: From 0.45 to 0.91 inch (1.1 to 2.3 cm).

Height: 0.2 inch (0.5 cm).

DOME SPACING

Standards Compliance: ADAAG 2.29.2 and ADA 705.1.2.

Center-to-center spacing: From 1.6 to 2.4 inches (4.1 to 6.1 cm).

Base-to-base spacing: 0.65 in (1.7 cm) minimum. Measured between the most adjacent domes on a square grid.

VENT HOLES

Release gasses and air for maximum concrete contact.

PLATFORM EDGES

Standards Compliance: ADAAG 2.29.2 and ADA 705.2.

Detectable warning surfaces at platform boarding edges to have a depth of 24 in (60.7 cm) and extend full length of the public use areas of the platform.

Lifting Brackets: Included for simplifying installation.

EXECUTION

EXAMINATION

Examine paving or other substrates for compliance with manufacturer’s requirements for placement and location of embedded items, condition of substrate, and other conditions affecting installation of detectable warning plates.

Proceed with installation only after unsatisfactory conditions have been corrected.

INSTALLATION

Drawing Coordination: Ensure that bicycle lockers, when in use, do not impede required paths of egress and accessible travel. Allow adequate clearance for bicycle storage and removal.

General: Comply with manufacturer’s installation requirements and setting drawings.

Do not install damaged, cracked, chipped, deformed, or marred detectable warning plates.

CLEANING & PROTECTION

Protect installed products until completion of project.

Touch-up, repair, or replace damaged products before Substantial Completion.

CLOSEOUT ACTIVITIES

Provide executed warranty.

END OF SECTION

Copyright 2019 by Reliance Foundry Co. Ltd.; qualified design and construction professionals may copy for preparation of construction specifications and purchase orders for Reliance Foundry products. Issued March 2019. Subject to change.

Issued July 2016. Subject to change.