Reliance Foundry Co Ltd

#207-6450 148 St

Surrey, BC V3S 7G7

Phone: 604-547-0460

Phone: 1-877-789-3245

Fax: 604-590-8875

Email: info@reliance-foundry.com

Website: www.reliance-foundry.com

Reliance Foundry is an award-winning supplier of high-quality stock and custom-designed site furnishings

for architectural, traffic management, and industrial applications. Reliance Foundry brings over 90 years of expertise to the design and manufacturing of our products.

Reliance Foundry’s crash-rated bollards have exceptional stopping power and impact resistance. They are certified to ASTM ratings with the ability to stop vehicles from intruding into pedestrian zones. Very easy installation for a crash-rated bollard, with no assembly or specialty subgrade required—simply set the prefabricated bollard into the site and pour concrete.

Specification Coordination: Edit this guide specification to meet project requirements. Coordinate with other specification sections as required. If using more than one bollard type, use the same designation in both drawings and specifications. Display hidden text to see or print notes to specifier.

Drawing Coordination: Show locations, sizes, layout, and spacing of each type of bollard or bollard cover required. Show footing and security post details, if required. Soft metric conversion is provided for all measurements.

SECTION 34 71 13.16

VEHICLE BARRIER SYSTEMS

GENERAL

SUMMARY

Section Includes:

Crash-rated bollards

[_________________]

Related Requirements:

[Section 03 30 00: Cast-in-Place Concrete]

[Section 28 13 63 - Access Control Vehicle Identification System]

[_________________]

REFERENCE STANDARDS

Department of Defense (DoD) approved and listed on Anti-Ram Vehicle Barrier List

SUBMITTALS

Manufacturer’s Product Data

Comply with Section [01 33 00 – Submittal Procedures.] [_________.]

Product Data: Provide for each type of bollard, component, finish, and accessory specified.

Maintenance Data: Submit manufacturer’s field touch-up, cleaning, and maintenance instructions.

Warranty Documentation: Submit sample of manufacturer’s warranty.

Setting Drawings: Show embedded items and cutouts required for work specified by other Sections.

QUALITY ASSURANCE

Comply with Section [01 43 00 – Quality Assurance.] [_________.]

DELIVERY, STORAGE AND HANDL-ING

Comply with Section [01 66 00 – Product Storage and Handling Requirements.] [_________.]

Protect bollards during delivery, storage, and handling.

WARRANTY

Comply with Section [01 78 36 – Warranties.] [_________.]

PRODUCTS

MANUFACTURERS

Manufacturer: Reliance Foundry Co. Ltd.

Phone: 604-547-0460 or 1-877-789-3245

Fax: 604-590-8875

Website: http://www.reliance-foundry.com/bollard

Email: info@reliance-foundry.com

Substitutions: [Comply with provisions of [Section 01 25 00 "Substitution Procedures"] [_________] for substitution procedures.] [Not allowed.]

CRASH-RATED BOLLARDS

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-04.

Height: 72 inches (182.9 cm)

Height Above Grade: 38 1/2 inches (97.8 cm)

Diameter: 8 inches (20.3 cm)

Weight: 410 lbs. (186 kg)

Design: M30 ASTM crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-04-R.

Height: 72 inches (182.9 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 8 inches (20.3 cm)

Weight: 410 lbs. (186 kg)

Design: Removable M30 crash engineered bollard.

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-04-SM

Height: 50 inches (127 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 8 5/8 inches (18.73 cm)

Weight: 780 lbs. (353.8 kg)

Design: Shallow Mount M30 ASTM crash-rated bollard (three-bollard array required).

Material: Steel

Country of Origin: United States

Finish: Paint coated.

Color Coating:

Type: Paint

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-12.

Height: 75.5 inches (191.8 cm)

Height Above Grade: 39-1/2 inches (100.3 cm)

Diameter: 10 inches (25.4 cm)

Weight: 1257 lbs. (570.2 kg)

Design M50 ASTM crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-12-R.

Height: 75.5 inches (191.8 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 10 inches (25.4 cm)

Weight: 1257 lbs. (570.2 kg)

Design: Removable M50 crash engineered bollard

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-40.

Height: 66 inches (182.9 cm)

Height Above Grade: 36 inches (91.4 cm)

Diameter: 6 5/8 inches (16.8 cm)

Weight: 190 lbs. (86.2 kg)

Design: C40 crash engineered bollard.

Material: Steel.

Country of Origin: United States.

Finish: Prime paint coated.

Color Coating:

Type: Prime paint.

Color: [Red]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-40-R.

Height: 66 inches (182.9 cm)

Height Above Grade: 36 inches (91.4 cm)

Diameter: 6 5/8 inches (16.8 cm)

Bollard Weight: 126 lbs. (57.2 kg)

Total Weight: 155 lbs. (70.3 kg)

Design: Removable C40 crash engineered bollard.

Material: Steel.

Country of Origin: United States.

Finish: Prime paint coated.

Color Coating:

Type: Prime paint.

Color: [Red]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S10.

Height: 65 inches (165.1 cm)

Height Above Grade: 32 inches (81.3 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 7/8 inches (14.9 cm)

Weight: 117 lbs. (53.1 kg)

Design: S10 P1 crash-rated bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S10-R.

Height: 65 inches (165.1 cm)

Height Above Grade: 32 inches (81.3 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 7/8 inches (14.9 cm)

Weight: 143 lbs. (64.9 kg)

Design: S10 P1 removable crash-rated bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S20.

Height: 69 inches (175.3 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 15/16 inches (15.1 cm)

Weight: 205 lbs. (93 kg)

Design: S20 P1 crash-rated bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S20-R.

Height: 69 inches (175.3 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 5/16 inches (15.1 cm)

Weight: 262 lbs. (118.8 kg)

Design: S20 P1 removable crash-rated bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S30.

Height: 78 inches (198.1 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 42 inches (106.7 cm)

Max Width: 8 3/16 inches (20.8 cm)

Weight: 340 lbs. (154.6 kg)

Design: S30 P2 crash-rated bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S30-R.

Height: 78 inches (198.1 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 42 inches (106.7 cm)

Max Width: 8 3/16 inches (20.8 cm)

Weight: 404 lbs. (183.3 kg)

Design: S30 crash engineered bollard.

Material: Steel.

Country of Origin: Vietnam.

Finish: Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; Halvard R-1051-M50-SR.

Height: 59 inches (159.9 cm)

Height Above Grade: 43 5/16 inches (110 cm)

Height Below Grade: 15 3/4 inches (40 cm)

Diameter: 14 inches (35.6 cm)

Weight: 285 lbs. (129.3 kg)

Design: M50 P1 Crash-Rated Bollard.

Material: Steel with 316 Stainless Steel Cover

Country of Origin: Turkey.

Finish: Hot Dip Galvanized

Installation:

Concrete Embedded, 3 Bollard Array

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S10 Patriot.

Height: 65 inches (165.1 cm)

Height Above Grade: 32 inches (81.3 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 7/8 inches (14.9 cm)

Weight: 128 lbs. (58 kg)

Design: S10 P1 crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S10-R Patriot.

Height: 65 inches (165.1 cm)

Height Above Grade: 32 inches (81.3 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 7/8 inches (14.9 cm)

Weight: 150 lbs. (68 kg)

Design: S10 P1 removable crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S20 Patriot.

Height: 69 inches (175.3 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 15/16 inches (15.1 cm)

Weight: 223 lbs. (101 kg)

Design: S20 P1 crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S20-R Patriot.

Height: 69 inches (175.3 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 33 inches (83.8 cm)

Max Width: 5 5/16 inches (15.1 cm)

Weight: 277 lbs. (125 kg)

Design: S20 P1 removable crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S30 Patriot.

Height: 78 inches (198.1 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 42 inches (106.7 cm)

Max Width: 8 3/16 inches (20.8 cm)

Weight: 340 lbs. (154 kg)

Design: S30 P2 crash-rated bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1010-S30-R Patriot.

Height: 78 inches (198.1 cm)

Height Above Grade: 36 inches (91.4 cm)

Height Below Grade: 42 inches (106.7 cm)

Max Width: 8 3/16 inches (20.8 cm)

Weight: 423 lbs. (192 kg)

Design: S30 crash engineered bollard.

Material: Steel.

Country of Origin: United States.

Finish: Black Primer, Hot-dipped Galvanized

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-04-SM-R.

Height: 51 1/2 inches (130.8 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 8 5/8 inches (21.9 cm)

Weight: 1020 lbs. (462.7 kg)

Design: Removable Shallow Mount M30 Crash Engineered Bollard

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-12-R.

Height: 75.5 inches (191.8 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 10 3/4 inches (27.3 cm)

Weight: 1555 lbs. (705.3 kg)

Design: Removable M50 Crash Engineered Bollard

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-12-SM.

Height: 52 7/8 inches (134.3 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 10 3/4 inches (27.3 cm)

Weight: 1965 lbs. (891.3 kg)

Design: Shallow Mount M50 Crash-Rated Bollard

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

Crash-rated bollard [Designer’s Designation]:

Model: Reliance Foundry; R-1009-12-SM-R.

Height: 52 7/8 inches (134.3 cm)

Height Above Grade: 39 1/2 inches (100.3 cm)

Diameter: 10 3/4 inches (27.3 cm)

Weight: 2235 lbs. (1013.8 kg)

Design: Removable Shallow Mount M50 Crash Engineered Bollard

Material: Steel.

Country of Origin: United States.

Finish: Paint coated.

Color Coating:

Type: Paint.

Color: [Black]

Installation:

Concrete Embedded

EXECUTION

EXAMINATION

Examine paving or other substrates for compliance with manufacturer’s requirements for placement and location of embedded items, condition of substrate, and other conditions affecting installation of bollards.

Proceed with installation only after unsatisfactory conditions have been corrected.

INSTALLATION

General: Comply with manufacturer’s installation instructions and setting drawings.

Do not install damaged, cracked, chipped, deformed or marred bollards. Field touch-up minor imperfections in accordance with manufacturer’s instructions. Replace bollards that cannot be field repaired.

CLEANING & PROTECTION

Normal cleaning may be done with clean water and a soft cloth. Detailed cleaning and inspection instructions are available at www.reliance-foundry.com/bollard/maintenance-bollards.

Protect bollards against damage.

Immediately prior to Substantial Completion, clean bollards in accordance with manufacturer’s instructions to remove dust, dirt, adhesives, and other foreign materials.

Touch up damaged finishes according to manufacturer’s instructions.

CLOSEOUT ACTIVITIES

Provide executed warranty.

END OF SECTION

Copyright 2020 by Reliance Foundry Co. Ltd.; qualified design and construction professionals may copy for preparation of construction specifications and purchase orders for Reliance Foundry products. Issued March 2020. Subject to change.

Issued July 2016. Subject to change.